Bit-Crushed 8-bit Synth

Raspberry Pi Pico - PWM Audio Build Guide

Plays a tone then intentionally reduces bit depth and/or sample rate. The grit and aliasing are the feature - arcade/chiptune output. HW is minimal; the interesting work is all in software.

Pico GPIO (PWM)  ->  voltage divider + RC filter  ->  LM386  ->  8 ohm speaker

Possible Scope

Two controls: pitch + crush amount. Two modes: normal and crushed. Stretch goal: tiny arpeggiator.

Code to write: timer ISR, tone gen, bit-shift quantization, downsample/sample-hold, mode switch.

Components

2.2 k / 1 k voltage divider (R1/R2), not equal RC resistors. This drops the 3.3 V Pico output to ~1 V before the LM386 input.

| Component | Value | Purpose | Note |
| --- | --- | --- | --- |
| Pico | - | MCU / brain | RP2040, 125 MHz default |
| R1 | 2.2 kO | Voltage divider top | Drops 3.3 V PWM to ~1 V for LM386 |
| R2 | 1 kO | Voltage divider bottom |  |
| C1 | 100 nF | LP filter + DC block | Across R2 - kills PWM carrier |
| LM386 | N-1 or N-3 | Audio amp | Power from 5 V VBUS |
| C2 | 10 uF elec. | Output coupling | LM386 out to speaker - blocks DC |
| C3 | 100 nF | Power decoupling | On LM386 VCC, close to IC |
| C4 | 10 uF elec. | Pin 7 bypass | Optional but cuts hiss noticeably |
| Speaker | 8 O | Audio output |  |

Extras: 1-2 pots (pitch + crush), 1 button (mode), USB power bank.

Simulation

Tinkercad does not support Pico. So I’ll be using Wokwi (runs in browser, supports MicroPython and C SDK, has a logic analyzer) to test things.

- Pico Audio - working PWM ISR project - (might fork this) PWM interrupt handler with adjustable sample rate

- Pico PWM + logic analyzer - have to verify frequency math before touching hardware

- New Pico MicroPython blank project

References

- moefh - Sound Output with the Raspberry Pi Pico - best schematic - breadboard photo + diagram (good starting point)

- moefh/pico-audio-demo (GitHub) - C code + schematic, 8-bit at 22050 Hz via PWM/DMA

- Greg Chadwick - PWM Audio (Pico Pt. 3) - clock divider math, DMA setup, sample-repeat trick

- RS Online - Pico Piano Pt. 3 - oscilloscope output shown at each RC filter stage

- rgrosset/pico-pwm-audio (GitHub) - minimal C tone generator, easy starting point

Approach

Will try to get a stable 440 Hz tone out of the speaker first - that confirms the hardware is solid. Everything after is a software problem.